2. Literature review

2.1 Concept of rainfall-runoff modeling

Rainfall–runoff modeling is an important component of hydrological science because it provides a quantitative framework for understanding how precipitation is transformed into runoff within a watershed. The relationship between rainfall and runoff is controlled by several factors, including rainfall intensity and duration, soil characteristics, antecedent moisture conditions, land use and land cover, topography, drainage characteristics, evapotranspiration, and surface-water storage. Because these factors vary spatially and temporally, the response of a watershed to a given rainfall event is rarely linear or uniform. Hydrological models are therefore widely used to simplify and represent these processes and to estimate streamflow from available meteorological and watershed information (Sahu et al., 2023).

Rainfall–runoff models have become increasingly important for flood forecasting, watershed management, water-resources planning and assessment of hydrological hazards. Recent research has produced a wide range of empirical and conceptual based data-driven models. However, model selection depends strongly on the purpose of the study, the characteristics of the watershed, the temporal and spatial resolution of available data, and the level of process representation required. Greater model complexity does not necessarily result in better performance because model uncertainty can arise from input data, parameterization, structural assumptions, and spatial heterogeneity (Sahu et al., 2023).

Among the available rainfall–runoff models, the Hydrologic Engineering Center–Hydrologic Modeling System (HEC-HMS) has been widely applied because of its flexible and modular structure. The model allows precipitation to be converted into runoff through different loss, transform, baseflow, and routing methods. An appropriate representation can be selected according to the characteristics and objectives of a particular watershed. Recent reviews have identified the Soil Conservation Service Curve Number (SCS-CN) and SCS Unit Hydrograph methods as among the most frequently used approaches in HEC-HMS applications, while recession methods and Muskingum-type routing approaches are also commonly applied (Labade et al., 2025; Sahu et al., 2023; Turkar et al., 2025).

The integration of HEC-HMS with Geographic Information Systems (GIS) has further increased its applicability. GIS enables the extraction of watershed boundaries, drainage networks, slopes, flow paths, land-use information, and soil characteristics from spatial datasets. These data can subsequently be used to develop hydrological parameters for the HEC-HMS model. Integrated ArcGIS-derived land-use, soil, and slope information with rainfall and runoff observations are used to develop HEC-HMS models for the Punpun River Basin (Ranjan & Singh, 2022). Their study demonstrated the practical value of combining spatial watershed information with observed hydrometeorological data for rainfall–runoff simulation.

The application of HEC-HMS is particularly relevant to Bangladesh because of the country’s monsoonal rainfall regime, extensive river network, low-lying floodplains, wetlands, and recurrent flooding. Several recent studies have applied HEC-HMS to Bangladesh river basins, including the Khowai, Bangali, Halda, Old Brahmaputra, Dhaka, and Atrai–Karatoa systems. These studies demonstrate the applicability of the model under Bangladesh’s hydrological conditions but also highlight challenges related to rainfall representation, discharge availability, baseflow simulation, and model parameterization (M. B. Haque et al., 2024; Moniruzzaman & Mahalder, 2026; Nur et al., 2022; Zhang et al., 2022).

The present study focuses on rainfall–runoff modelling of the Gumai Beel–Ichamoti River system using HEC-HMS. Gumai Beel represents a low-lying wetland and agricultural environment, while the Ichamoti River forms an important drainage component of the surrounding landscape. The hydrological response of such a system may differ from the conventional upland watersheds. Because wetland storage, seasonal inundation, drainage connectivity, and agricultural land use can influence the timing and magnitude of runoff. Therefore, a review of rainfall–runoff processes, hydrological modelling approaches, HEC-HMS methodology, GIS integration, wetland hydrology, and previous Bangladesh applications is necessary to establish the scientific basis for the present research.

2.2 Rainfall-runoff process

Rainfall–runoff transformation governs the hydrological routing of precipitation across a drainage basin and its ultimate conversion into streamflow discharge. Incident precipitation across the land surface is partitioned dynamically among canopy interception, infiltration into the vadose zone, surface depression storage, evapotranspiration loss, and overland or subsurface lateral flow. The volumetric fraction of precipitation that reaches the channel network is dictated by the dynamic equilibrium among these constituent processes. Consequently, the hydrograph configuration produced by a given precipitation event is governed not only by gross rainfall depth, but also by storm duration, temporal hyetograph distribution, peak rainfall intensity, and the antecedent hydrological state of the catchment (Labade et al., 2025).

Precipitation intensity directly regulates the threshold between infiltration and direct overland flow. When precipitation rates surpass the saturated hydraulic conductivity and infiltration capacity of the soil, infiltration-excess (Hortonian) overland flow is initiated and rapidly routed to the channel network. Conversely, rainfall rates below the infiltration threshold primarily replenish soil moisture storage or contribute to subsurface matrix flow. Antecedent moisture conditions further modulate this response: a dry catchment exhibits substantial initial abstraction and retention capacity, whereas antecedent saturation markedly reduces soil matrix suction potential, accelerating saturation-excess overland flow. As a result, identical storm depths falling under differing initial moisture conditions generate fundamentally disparate runoff hydrographs (Shi & Wang, 2020).

Catchment physiography and surface cover impose critical boundary controls on hydrograph translation and attenuation. Parameters such as soil texture, effective porosity, hydraulic conductivity, soil profile depth, topographic gradient, drainage density, and surface roughness collectively determine the velocity and storage dynamics of routed water. Anthropogenic land-use and land-cover (LULC) modifications directly influence these hydrologic controls such as urban expansion increases surface imperviousness and accelerates hydrograph. Dense vegetative canopies enhance interception and infiltration capacity. In agricultural floodplains, hydrological responses exhibit pronounced seasonal variability driven by cyclical crop growth stages, tillage practices, and artificial field drainage regimes (Dibaba et al., 2020).

In low-lying wetland such as Gumai Beel, runoff generation is influenced by dynamic depression storage, delayed infiltration, and temporary surface-water retention. Within the floodplain morphology of Bangladesh, river–wetland systems function as hydrodynamic buffers whose storage behavior is dictated by seasonal monsoon inundation pulses and dynamic lateral river connectivity (Adnan et al., 2020; Akter & Sawon, 2024). These seasonal fluctuations in backwater staging, storage volume, and hydraulic connectivity introduce significant attenuation and lag into the catchment discharge hydrograph (Akter & Sawon, 2024). It represents a critical hydrodynamic mechanism in the rainfall–runoff modeling of the Gumai Beel–Ichamoti River system.

2.3 Classification and selection of hydrological models

Hydrological models are conventionally differentiated along two principal axes, namely the degree of physical process representation and the level of spatial discretisation. Empirical or data-driven models establish statistical transfer functions between input and output series without explicit reference to physical mechanisms, conceptual models represent the catchment as a cascade of interconnected storage elements whose governing equations approximate physical behaviour, and physically based models solve conservation equations for mass, momentum, and energy using parameters that are in principle independently measurable. With respect to spatial resolution, lumped models treat the catchment as a single homogeneous unit, semi-distributed models subdivide it into sub-basins within which parameters are averaged, and fully distributed models resolve hydrological state variables on a regular computational grid (Sahu et al., 2023). HEC-HMS occupies an intermediate position within this taxonomy, functioning as a conceptual and semi-distributed system that reproduces the dominant runoff-generation mechanisms without imposing the data requirements associated with fully distributed formulations (Turkar et al., 2025).

The proliferation of hydrological models is driven only partially by genuine differences in application context, since a substantial component of model diversity reflects institutional familiarity, accessibility of source code, and the reuse of pre-existing model configurations. The rationale for model selection is rarely articulated in published studies, and the adequacy of a given model for its landscape and purpose is frequently left unaddressed (Horton et al., 2022). Model choice must therefore be justified explicitly with reference to catchment characteristics, the availability and resolution of input data, and the specific objectives of the investigation, rather than adopted by convention.

For data-limited catchments, the operative comparison in contemporary practice lies between conceptual semi-distributed systems such as HEC-HMS and more heavily parameterised ecohydrological frameworks such as the Soil and Water Assessment Tool (SWAT). Both frameworks are capable of reproducing observed streamflow with acceptable accuracy under conditions of limited observational support (Aliye et al., 2020). In a comparative application to the Atrai–Karatoa River Basin of northern Bangladesh, SWAT reproduced high flows more faithfully whereas HEC-HMS achieved superior accuracy across the medium-flow range, with both models simulating low flows adequately (Moniruzzaman & Mahalder, 2026). HEC-HMS accordingly constitutes a defensible selection where the modelling objective is hydrograph generation rather than the simulation of sediment transport, nutrient flux, or land-management scenarios, and where the parsimony of its parameter set corresponds to the extent of the available observational record.

A further operational distinction separates event-based from continuous simulation. Event-based configurations reproduce the catchment response to discrete storm episodes and are appropriate for design-flood estimation, peak discharge analysis, and flood forecasting, whereas continuous configurations track soil-moisture accounting through alternating wet and dry periods over multi-annual horizons and are appropriate for water-balance and water-resources assessment. The SCS-CN loss method dominates event-based practice, while the Soil Moisture Accounting (SMA) algorithm is preferred for continuous simulation (Labade et al., 2025; Turkar et al., 2025). Because the inundation regime of the Gumai Beel is governed simultaneously by discrete monsoon storm events and by the strongly seasonal water balance of the surrounding floodplain, the present study requires a modelling system capable of operating in both modes, a requirement satisfied by HEC-HMS within a single computational framework.

2.4 Structure and components of the HEC-HMS model

HEC-HMS is designed to simulate the complete precipitation–runoff process of dendritic watershed systems, and a model project comprises three principal components (U.S. Army Corps of Engineers [USACE], n.d.). The basin model represents the physical catchment as a network of hydrological elements including sub-basins, reaches, junctions, reservoirs, diversions, sources, and sinks. The meteorological model assigns precipitation and evapotranspiration boundary conditions to each sub-basin through gauge weighting, inverse-distance interpolation, or gridded input. The control specifications define the temporal window and computational time step of the simulation. Within each sub-basin, one algorithm is selected from each of several interchangeable method libraries governing canopy and surface storage, infiltration loss, direct-runoff transformation, and baseflow, while each routing reach is assigned an independent channel-routing method (Sahu et al., 2023; USACE, n.d.). This modular architecture accounts for the adaptability of the model across climatic regimes and explains its widespread adoption in regions where proprietary modelling systems are financially inaccessible (Hamdan et al., 2021).

2.4.1 Loss methods

The loss model determines the proportion of incident precipitation abstracted through infiltration, interception, and surface retention, and therefore the residual depth of precipitation excess available for direct runoff. The SCS-CN method remains the dominant formulation within the HEC-HMS application literature because it consolidates the combined influence of hydrologic soil group, land use and treatment, surface condition, and antecedent moisture into a single dimensionless parameter derivable from mappable catchment attributes (Labade et al., 2025; Soulis, 2021). The persistence of the method after six decades of application is attributable to its conceptual simplicity, well-documented input requirements, and transferability to ungauged catchments, although several limitations remain unresolved. Foremost among these is the initial abstraction ratio, for which locally calibrated values are frequently found to lie substantially below the conventional figure of 0.2 and often approach 0.05 or lower, implying systematic overestimation of initial losses when the standard ratio is applied uncritically (Soulis, 2021).

Extensions of the method have sought to incorporate variables absent from its original formulation. A modified SCS-CN equation combining the tabulated curve number with explicit slope-gradient, soil-moisture, and storm-duration factors raised model efficiency to approximately 80% during both calibration and validation and reduced root-mean-square error from 5.53 mm to 2.01 mm relative to the standard method, with soil-moisture parameters identified as the most influential, followed by storm duration and slope, and the initial abstraction ratio as the least sensitive (Shi & Wang, 2020). These findings carry direct methodological implications for the present study, in which antecedent moisture within a seasonally saturated beel environment varies far more widely than in the agricultural watersheds from which the original curve-number tabulations were derived.

Alternatives to the curve-number approach available within HEC-HMS include the initial-and-constant and deficit-and-constant loss methods, the Green–Ampt infiltration model, and the multi-layer SMA algorithm intended for continuous simulation (USACE, n.d.). Empirical evidence indicates reliable performance of the SCS-CN method in monsoonal and semi-arid event simulation, as demonstrated for the Al-Adhaim catchment of northern Iraq (Hamdan et al., 2021) and the Punpun River Basin of eastern India (Ranjan & Singh, 2022), whereas the SMA formulation is generally preferred for multi-annual continuous applications (Jawad, 2024). A consistent finding across this literature is that the curve number constitutes the single most sensitive parameter of the model. Curve number, infiltration rate, lag time, and baseflow have been identified as the controlling parameters across the reviewed body of applications (Turkar et al., 2025), and simulated runoff in the Halda catchment responded strongly to sub-basin curve numbers determined by the accuracy of the underlying land-cover classification (M. B. Haque et al., 2024).

2.4.2 Transform methods

The transform model converts the precipitation excess computed by the loss model into a direct-runoff hydrograph at the sub-basin outlet. Most applications employ unit-hydrograph theory, for which HEC-HMS provides the SCS dimensionless unit hydrograph parameterised by basin lag time, the Clark unit hydrograph which represents translation and attenuation separately through a time–area histogram and a linear-reservoir storage coefficient, and the Snyder synthetic unit hydrograph parameterised by lag and peaking coefficients (USACE, n.d.). The SCS unit hydrograph is the most frequently adopted transform across international practice and is typically paired with SCS-CN losses (Labade et al., 2025; Sahu et al., 2023), a configuration applied successfully in a range of monsoonal and semi-arid settings (Hamdan et al., 2021; Nujhat et al., 2024; Ranjan & Singh, 2022). The Clark formulation has been preferred in several large-basin continuous applications, in which storage coefficient and time of concentration are treated as primary calibration parameters (Jawad, 2024).

For extremely flat catchments the estimation of lag time warrants particular caution. Empirical lag equations were predominantly derived from sloping terrain, and any misestimation propagates directly into errors in the timing of the simulated hydrograph peak. In floodplain environments where topographic gradients approach the vertical resolution of the available elevation data, calibration of lag and storage parameters against observed hydrographs becomes indispensable rather than optional, an approach reflected in recent optimisation strategies for low-gradient and large-basin applications (Admas et al., 2025; Jawad, 2024).

2.4.3 Baseflow methods

Baseflow representation constitutes the most frequently underperforming component of event-oriented hydrological models, yet in floodplain-wetland systems the slow-drainage component may dominate both the recession limb of the hydrograph and the dry-season water balance. Recession, bounded-recession, linear-reservoir, and constant-monthly baseflow formulations are available within HEC-HMS (USACE, n.d.). The recession method is the most widely adopted in international practice owing to its numerical stability and its realistic representation of groundwater depletion (Labade et al., 2025), whereas the linear-reservoir formulation with multiple groundwater layers has been preferred in continuous large-basin applications (Jawad, 2024).

Recent Bangladeshi experience indicates that baseflow simulation represents a genuine methodological difficulty rather than a routine calibration exercise. Satisfactory aggregate performance statistics have been obtained alongside a poor correspondence between observed and simulated flows along the baseflow portion of the hydrograph, a discrepancy attributed to unrepresented groundwater–surface water exchange and for which offline coupling with a groundwater model has been recommended (M. B. Haque et al., 2024). Groundwater delay time and the baseflow alpha factor have similarly been identified among the most sensitive parameters governing simulation quality in the Upper Halda Basin (Raihan et al., 2020). These findings caution against interpreting event-calibrated models as complete descriptions of low-flow behaviour, a caution of particular force in beel environments where monsoon storage is released gradually throughout the post-monsoon recession.

2.4.4 Channel routing methods and hydraulic coupling

Flow routing through channel reaches is available in HEC-HMS through the Muskingum, Muskingum–Cunge, kinematic-wave, modified-Puls, and lag methods (USACE, n.d.). The Muskingum method, which conceptualises a reach as a linear storage element governed by a travel-time parameter and a dimensionless weighting parameter, remains the most commonly adopted routing approach in applications comparable to the present study (Hamdan et al., 2021; Labade et al., 2025; Nujhat et al., 2024). Its principal limitation, shared by all hydrologic routing schemes, is the inability to represent backwater effects, flow reversal, or looped stage–discharge relationships, all of which are characteristic of extremely flat deltaic channels in which downstream water level rather than upstream inflow governs conveyance.

Where such hydraulic controls are material, contemporary practice couples HEC-HMS with the Hydrologic Engineering Center River Analysis System (HEC-RAS). In an integrated flood-risk framework for the Old Brahmaputra River floodplain of Bangladesh, HEC-HMS supplied inflow hydrographs to coupled one- and two-dimensional HEC-RAS models, yielding Nash–Sutcliffe efficiency values of 0.93 and 0.81 and percent bias of −1.17% and 2.40% during calibration and validation respectively (Zhang et al., 2022). An equivalent coupling applied to the Gumara River in the Upper Blue Nile Basin quantified inundation extents for return periods between 2 and 100 years and evaluated the hydraulic effect of dyke construction (Admas et al., 2025), and the same architecture has been used to generate flood inundation maps along the lowermost reach of the Brahmaputra (Jawad, 2024). Because drainage of the Gumai Beel is regulated in part by water levels within the receiving Ichamoti channel rather than by catchment inflow alone, an analogous coupling represents the logical methodological extension of the present work.

2.5 Integration of GIS and remote sensing in hydrological modelling

2.5.1 Digital elevation models and watershed delineation

Semi-distributed modelling is initiated through terrain analysis, in which sub-basin boundaries and stream networks are delineated from a digital elevation model (DEM) and physiographic attributes such as drainage area, mean slope, and longest flow path are extracted for each computational unit. A widening family of freely available global elevation products, including SRTM, NASADEM, ASTER, AW3D30, MERIT, and TanDEM-X, now competes for this role, and the differences among them are hydrologically consequential. Vertical accuracy varies substantially among products, and terrain slope exerts the strongest control on error magnitude, with AW3D30 exhibiting the most stable performance across geographic settings and NASADEM offering only marginal improvement relative to its SRTM predecessor (Uuemaa et al., 2020).

The reliability of automated delineation deteriorates precisely within the terrain class represented by the present study area. Delineation outcomes are materially conditioned by the selection of DEM product, its spatial resolution, and the contributing-area threshold adopted for stream definition, and these choices propagate into sub-basin geometry and all subsequently derived parameters (Datta et al., 2022). In low-relief deltaic terrain, where total elevation range may amount to only a few metres and where anthropogenic features such as roads, embankments, and sluice structures rather than natural topography govern actual flow pathways, DEM vertical error can exceed the topographic signal that the delineation algorithm is intended to detect. Systematic verification of automatically delineated drainage against hydrographic maps and field observation is therefore recommended (Datta et al., 2022), and this procedure is adopted in the methodology of the present study.

2.5.2 Land use, soil data, and curve number derivation

Parameterisation of the loss model in sparsely gauged basins depends upon thematic spatial mapping. The established workflow intersects a land-use and land-cover classification, commonly derived from Landsat or Sentinel-2 imagery, with a hydrologic soil group layer obtained from national soil surveys or global soil databases, and assigns area-weighted composite curve numbers to each sub-basin through standard lookup tabulations (Ranjan & Singh, 2022; Turkar et al., 2025). Implementation of this procedure within ArcGIS for the Punpun River Basin, in which land-use, soil, and slope layers were generated and curve numbers computed as direct HEC-HMS input, produced coefficient of determination and Nash–Sutcliffe efficiency values exceeding 0.75 for monthly and monsoonal models with percent bias below 10% (Ranjan & Singh, 2022).

The sensitivity of simulated runoff to this parameterisation imposes a corresponding requirement for classification accuracy, since curve number ranks among the four most sensitive parameters across the reviewed applications (Turkar et al., 2025) and the rainfall–runoff relationship of the Halda catchment responded strongly to sub-basin curve numbers determined by land-cover classification (M. B. Haque et al., 2024). Remote sensing evidence further documents the rapidity of land-cover transformation within Bangladeshi wetland environments, where multi-decadal Landsat analysis has quantified substantial conversion of wetland and vegetation classes to agricultural and settlement uses (Bhattacharjee et al., 2021). This consideration assumes particular importance for beel catchments, in which the land surface alternates seasonally among open water, cultivated land, and marsh vegetation, such that a single-date classification cannot adequately represent the hydrological character of the catchment across the annual cycle.

2.5.3 Precipitation inputs in data-scarce catchments

Precipitation constitutes the dominant forcing variable of any rainfall–runoff model, and its estimation represents the principal source of predictive uncertainty where rain-gauge networks are sparse or discontinuous. Two families of alternatives to gauge interpolation have reached operational maturity, namely satellite multi-sensor products, foremost the Integrated Multi-satellitE Retrievals for Global Precipitation Measurement (IMERG), and atmospheric reanalyses, foremost the fifth-generation European Centre for Medium-Range Weather Forecasts reanalysis (Hersbach et al., 2020). Satellite products represent regional precipitation patterns and spatial means reliably and improve systematically with successive algorithm versions, while performing more accurately at monthly and annual aggregations than at daily and sub-daily resolutions and retaining recognised limitations for extreme intensities and orographically complex terrain (Pradhan et al., 2022).

Operational experience confirms that satellite and reanalysis forcing can support credible hydrological simulation even in large and sparsely instrumented basins. Coupled HEC-HMS and HEC-RAS models of the transboundary Brahmaputra Basin forced with near-real-time IMERG and GSMaP products, corrected against ground observations where these were available, produced acceptable discharge simulation and flood inundation mapping at Bahadurabad (Jawad, 2024). Because the present study can draw upon Bangladesh Water Development Board and Bangladesh Meteorological Department observations within and adjacent to the Pabna region, this literature supports a strategy of gauge-primary forcing supplemented by reanalysis products for gap-filling and internal consistency verification.

2.6 Model calibration, validation, and performance evaluation

Credible application of a rainfall–runoff model requires a disciplined testing protocol in which model parameters are estimated against one portion of the observational record and predictive skill is demonstrated against an independent portion. This split-sample framework is followed, at least in its elementary form, by essentially all of the applications reviewed in this chapter (Hamdan et al., 2021; Nujhat et al., 2024; Ranjan & Singh, 2022; Zhang et al., 2022). The framework itself has nevertheless been subjected to recent re-examination. In a large-sample experiment spanning 463 catchments, two conceptual hydrological models, and 50 alternative data-splitting schemes, the widespread practice of calibrating against older records and validating against more recent ones consistently degraded subsequent predictive performance, and calibration against the full available record proved the most robust strategy for operational application (Shen et al., 2022). In a thesis context, where independent demonstration of predictive skill remains an examination requirement, these findings support the reporting of both a conventional split-sample evaluation and a final parameter set re-estimated across the complete record.

Quantitative evaluation of model performance rests upon a compact set of statistical indicators. Nash–Sutcliffe efficiency, the coefficient of determination, and root-mean-square error constitute the most frequently reported metrics across HEC-HMS applications (Labade et al., 2025), with percent bias and the ratio of root-mean-square error to the standard deviation of observations increasingly reported alongside them (Ranjan & Singh, 2022). The interpretation of these indicators is not mechanical, however. The goodness-of-fit criterion adopted as the objective function materially determines the resulting parameter set, and criteria should therefore be matched deliberately to the modelling purpose, with peak-weighted criteria appropriate to flood design and volume-oriented criteria appropriate to water-balance assessment (Althoff & Rodrigues, 2021). It follows that reporting a single aggregate efficiency score, detached from the objective function used to obtain it, provides an incomplete account of model performance.

Calibration procedures range from manual adjustment guided by physical reasoning to fully automated optimisation. Univariate-gradient and simplex search algorithms are available within HEC-HMS in combination with a selection of objective functions (USACE, n.d.), and the reviewed applications span this full methodological range, encompassing manual calibration informed by parameter plausibility (Nujhat et al., 2024), automated optimisation using gradient search (Jawad, 2024), and staged procedures in which formal sensitivity analysis precedes calibration in order to concentrate effort upon the most influential parameters (Admas et al., 2025). The consistent identification of curve number, lag time, and routing parameters as the dominant controls upon model response (Turkar et al., 2025) provides a defensible starting parameter set for the present study. Reporting of calibrated parameter ranges alongside point estimates is further warranted, since alternative objective functions may select materially different yet equally acceptable parameter combinations (Althoff & Rodrigues, 2021).

2.7 Applications of HEC-HMS in international practice

The international application literature published since 2020 is extensive, and a selective review of methodologically instructive studies is presented here, with the principal characteristics of each summarised in Table 2.1. A daily-timestep model of the semi-arid Al-Adhaim River catchment in northern Iraq, constructed using HEC-GeoHMS terrain preprocessing together with the SCS-CN, SCS unit hydrograph, and Muskingum configuration and calibrated across two hydrological years with verification across a third, returned coefficients of determination of approximately 0.90 for both phases and proved suitable for reservoir inflow estimation, although simulated dam discharge was slightly overestimated (Hamdan et al., 2021). A comparable configuration applied to the Punpun River Basin of eastern India across the period 2005 to 2017, in which parallel daily, monthly, and monsoonal formulations were developed, demonstrated that temporal aggregation materially improved performance, with the monthly model outperforming both alternatives (Ranjan & Singh, 2022).

Comparative evaluation of HEC-HMS against SWAT for a data-scarce catchment of the Ethiopian Rift Valley Lakes Basin established the viability of the lighter-parameterised framework under conditions of limited observational support (Aliye et al., 2020). The Ethiopian experience has since been extended toward integrated flood-risk analysis, in which HEC-HMS design hydrographs were coupled with two-dimensional HEC-RAS hydraulics for the Gumara River floodplain in order to quantify the reduction in inundated area achieved through dyke construction across return periods from 2 to 100 years (Admas et al., 2025). Three generalisations emerge from this body of work. First, the combination of SCS-CN loss, SCS unit hydrograph transform, and Muskingum routing constitutes the effective default configuration for event-scale application and performs satisfactorily across monsoonal and semi-arid regimes (Hamdan et al., 2021; Labade et al., 2025). Second, the curve number is almost invariably the most sensitive parameter, such that its spatial estimation and calibration bounds govern overall model quality (Turkar et al., 2025). Third, model performance is constrained less by algorithmic structure than by the quality of input data, and by precipitation representation above all (Jawad, 2024; Pradhan et al., 2022).

Table 2.1

Summary of Selected HEC-HMS Applications Reviewed in This Chapter

| Study | Study area | Methods adopted | Reported performance |
| --- | --- | --- | --- |
| Aliye et al. (2020) | Rift Valley Lakes Basin, Ethiopia | HEC-HMS and SWAT comparison | Both models applicable in data-scarce conditions |
| Raihan et al. (2020) | Upper Halda Basin, Bangladesh | SWAT (comparative baseline) | R² = 0.80; NSE = 0.71 |
| Hamdan et al. (2021) | Al-Adhaim catchment, Iraq | SCS-CN; SCS UH; Muskingum; HEC-GeoHMS | R² ≈ 0.90 (calibration and verification) |
| S. Haque et al. (2021) | Brahmaputra Basin, Bangladesh | Continuous simulation; MUSLE sediment routing | NSE = 0.65 (cal.); 0.54 (val.) |
| Ranjan & Singh (2022) | Punpun River Basin, India | SCS-CN; ArcGIS-derived CN; daily/monthly/monsoonal | R² and NSE > 0.75; PBIAS < 10% |
| Zhang et al. (2022) | Old Brahmaputra floodplain, Bangladesh | HEC-HMS coupled with HEC-RAS 1D/2D | NSE = 0.93 (cal.); 0.81 (val.) |
| Nur et al. (2022) | Khowai River Basin, Bangladesh | Event-based HEC-HMS simulation | Model applicable to flash-flood-prone basin |
| Jawad (2024) | Brahmaputra Basin (transboundary) | SMA; Clark UH; linear reservoir; IMERG/GSMaP; HEC-RAS | Credible discharge and inundation from satellite forcing |
| Nujhat et al. (2024) | Gumti River Basin, Bangladesh | SCS-CN; Muskingum; SRTM delineation | R² = 0.64 (cal.); 0.68 (val.); PBIAS very good |
| M. B. Haque et al. (2024) | Halda River catchment, Bangladesh | SCS-CN optimised against SWAT-derived values | NSE = 0.72 (cal.); 0.82 (val.); baseflow underestimated |
| Admas et al. (2025) | Gumara River, Upper Blue Nile Basin | HEC-HMS coupled with HEC-RAS; return-period analysis | Inundation quantified for 2–100-year events |
| Moniruzzaman & Mahalder (2026) | Atrai–Karatoa River Basin, Bangladesh | HEC-HMS and SWAT comparison | HEC-HMS R² = 0.70 (cal.); 0.56 (val.) |

Note. NSE = Nash–Sutcliffe efficiency; R² = coefficient of determination; PBIAS = percent bias; CN = curve number; UH = unit hydrograph; SMA = soil moisture accounting; cal. = calibration; val. = validation.

2.8 Applications of HEC-HMS in Bangladesh

Bangladesh occupies the terminal reach of the Ganges–Brahmaputra–Meghna system, and its hydrological regime is characterised by intense monsoonal rainfall, transboundary inflow, extremely low topographic relief, and an extensive network of floodplain depressions. Within this setting, HEC-HMS has been adopted increasingly for basin-scale rainfall–runoff investigation, and the record published since 2020 now spans the principal basin types of the country.

At the scale of the major transboundary systems, a continuous HEC-HMS model of the poorly gauged Brahmaputra Basin calibrated against daily runoff at Bahadurabad for the period 1983 to 1996 and validated for 1997 to 2010 was extended with the Modified Universal Soil Loss Equation and Engelund–Hansen sediment routing in order to project future sediment load under the RCP8.5 scenario, yielding projected increases in mean annual sediment load of 34%, 67%, and 115% by the 2020s, 2050s, and 2080s respectively (S. Haque et al., 2021). For the Old Brahmaputra River floodplain, coupling of HEC-HMS with one- and two-dimensional HEC-RAS models supported an integrated assessment of flood risk, hazard, and vulnerability, with Nash–Sutcliffe efficiency values of 0.93 and 0.81 obtained during calibration and validation and the resulting framework proposed as a basis for early-warning systems and adaptation planning (Zhang et al., 2022).

At the sub-basin scale, event-based HEC-HMS simulation has been applied to the Khowai River Basin, a flash-flood-prone catchment of north-eastern Bangladesh (Nur et al., 2022). A model of the Gumti River Basin calibrated and validated using observations from 2019 to 2021 with SRTM-based catchment delineation, Muskingum routing, and SCS-CN losses returned coefficients of determination of 0.64 and 0.68 for calibration and validation respectively with percent bias in the very good performance category, and was recommended as a planning instrument for flood prediction and water-resources management (Nujhat et al., 2024). In the ecologically significant Halda catchment, curve numbers optimised against values derived from a companion SWAT application produced Nash–Sutcliffe efficiency values of 0.72 and 0.82, although underestimation of dry-season baseflow was explicitly documented (M. B. Haque et al., 2024). Comparison of HEC-HMS with SWAT for the Atrai–Karatoa River Basin of northern Bangladesh, a system hydrologically contiguous with the greater Chalan Beel floodplain to which the Pabna beel network belongs, indicated that SWAT captured high flows more accurately while HEC-HMS performed better across medium flows, with HEC-HMS achieving coefficients of determination of 0.70 and 0.56 during calibration and validation (Moniruzzaman & Mahalder, 2026).

Investigations employing alternative modelling frameworks complete the national picture. Streamflow simulation of the Upper Halda Basin using SWAT achieved a coefficient of determination of 0.80 and Nash–Sutcliffe efficiency of 0.71, with groundwater delay time, baseflow alpha factor, and curve number identified as the parameters exerting greatest influence upon performance and with the difficulty of representing spatial rainfall variability from a single gauging station emphasised (Raihan et al., 2020). Hydrological models of selected flash-flood-prone rivers have advanced the understanding of rapid-response catchment behaviour under monsoonal forcing (Akter & Sawon, 2024), while satellite-forced coupled hydrologic–hydraulic modelling has been demonstrated for the Bangladeshi reach of the Brahmaputra (Jawad, 2024).

Considered collectively, the Bangladeshi HEC-HMS literature exhibits a pronounced spatial concentration. Published applications cluster within large transboundary basins, flashy piedmont and hill catchments, and the Atrai–Karatoa corridor. The moribund distributary systems of the Ganges right bank, of which the Ichamoti River of Pabna constitutes a representative example, together with their associated beel catchments, have not been the subject of any published HEC-HMS rainfall–runoff investigation. It is precisely within such systems, where local rainfall accumulation and drainage congestion rather than upstream flood waves govern inundation behaviour, that quantification of runoff is most necessary for drainage design, and yet where the observational network is thinnest.

2.9 Wetland and beel hydrology in the Bangladesh floodplain

Wetland hydrology differs fundamentally from upland catchment hydrology in that the water regime, understood as the seasonal pattern of water-level rise, persistence, and recession, functions as the master variable governing biogeochemical processes, ecological structure, and land-use viability. Altered hydrology is correspondingly the principal pathway through which environmental and anthropogenic change degrades wetland function, since modification of water depth and residence time alters the balance between organic-matter production and decomposition and may convert a wetland from a net sink to a net source of carbon and nutrients (Salimi et al., 2021). Within the floodplain morphology of Bangladesh, the characteristic wetland landform is the beel, a saucer-shaped depression retaining water perennially or seasonally and receiving inflow both from direct rainfall–runoff generated within its local catchment and from lateral spill originating in adjacent river channels during the monsoon.

The hydrological functioning of these depressions has been substantially modified by embankment and polder infrastructure constructed from the 1960s onward. In the embanked delta of south-western Bangladesh, enclosure generates internal drainage congestion through a systemic mechanism in which embankments sever the hydraulic connection between floodplain and river channel, sediment accumulates within riverbeds rather than upon the floodplain surface, riverbed elevation rises relative to the enclosed land, gravity drainage through sluice structures progressively fails, and monsoon runoff accumulates within the beels as pluvial flooding (Adnan et al., 2020). A large majority of agricultural and aquacultural land within the embanked region now lies inside flood-susceptible zones, and controlled sediment reintroduction across 106 candidate beels has been evaluated as a rehabilitation strategy capable of raising land elevation by up to 1.4 m over five years (Adnan et al., 2020). The mechanism is structural and regional rather than locally anomalous, and it applies with equal force to the embanked interior floodplains of the Ganges right bank.

A second control operates at basin scale through the long-term reduction of dry-season flow within the Ganges distributary network. For the Gorai River, the principal Ganges distributary and the closest regional analogue to the Ichamoti, mean annual flow during 2000 to 2016 was approximately 13% lower than during 1984 to 1999, mean monthly high flow declined by 20%, and the river now frequently fails to satisfy an environmental flow requirement estimated at 295 m³/s, or 29% of mean annual flow, with a deficient-flow condition persisting from December through May (Ali & Hasan, 2022). Reduced discharge within parent channels deprives distributary offtakes of the sediment-flushing flows required to maintain conveyance capacity, initiating progressive siltation and eventual hydraulic disconnection of distributary systems.

The consequences of these combined mechanisms are documented directly for the wetland complex surrounding the study area. Five decades of Landsat imagery over Chalan Beel, the largest wetland ecosystem of Bangladesh and the system into which the Pabna floodplain drains, reveal a consistent trajectory of expanding farmland and developed land accompanied by declining vegetation and wetland extent, with the most pronounced transformation occurring between 2013 and 2023 (Yankyera & Alam, 2025). An equivalent trajectory has been reported for a north-eastern wetland ecosystem (Bhattacharjee et al., 2021). These transformations are hydrologically significant because reduction in wetland storage capacity diminishes the attenuation that beels provide to monsoon runoff, thereby increasing peak discharge and inundation depth within the residual drainage network.

2.10 The Ichamoti River and Gumai Beel system

The Ichamoti River of Pabna District exemplifies the condition described in the preceding section. The river originates from the Padma in the vicinity of Shibrampur within Pabna Sadar Upazila and follows a course of approximately 82 to 84 km through Pabna town toward the Hurasagar system in Bera Upazila. Over recent decades the channel has lost effective hydraulic connectivity with its parent rivers and has become progressively constricted through siltation, encroachment, solid-waste disposal, and aquatic weed infestation. The river bisects Pabna Municipality but no longer functions as an effective drainage outfall, being choked with water weeds and accumulated sediment, and blocked or undersized drains, absent operation and maintenance provision, and an unplanned drainage network have been identified by residents as the leading causes of inundation following heavy rainfall (Parvez et al., 2021). Approximately 7.9 km of the Ichamoti channel lies within the municipal area alongside 11.29 km of primary drains and 56.23 km of secondary drains, which establishes the scale of the drainage system dependent upon the river as outfall (Parvez et al., 2021).

Recognition of this condition has prompted large-scale government intervention. A rejuvenation programme valued at Tk 1,554.90 crore, implemented under the Bangladesh Water Development Board, provides for dredging across a 33.77 km reach of the river together with 44.07 km of connecting canals, 20 km of the Sutikhali River, and 12.37 km of the Bharara channel, with the stated objective of restoring hydraulic connectivity between the Ichamoti and the Padma and Jamuna systems (“Tk 1,554cr Project,” 2024). The hydrological design basis for such intervention requires quantitative estimation of the runoff volumes and hydrograph characteristics that the restored corridor must convey, which is precisely the information supplied by a calibrated rainfall–runoff model.

The Gumai Beel, which constitutes the focus of the present investigation, forms one component of the beel network of the Pabna floodplain draining through the Ichamoti corridor. Its inundation behaviour is governed by the interaction of three controls identified in the reviewed literature as characteristic of embanked distributary floodplains, namely local monsoon rainfall–runoff generated within the beel catchment, the conveyance capacity of the silted Ichamoti channel and its associated khals, which determines the rate at which stored water may be evacuated, and the stage of the receiving river system, which may impose backwater constraints upon gravity drainage (Adnan et al., 2020; Parvez et al., 2021). A rainfall–runoff model of the beel catchment constitutes the necessary first element of any quantitative analysis of this system, since it supplies the inflow boundary condition required for drainage design, for evaluation of the hydrological benefits attributable to the rejuvenation programme, and for assessment of waterlogging risk under current and projected rainfall regimes.

2.11 Research gap and summary

The literature reviewed in this chapter supports four conclusions that collectively establish the position of the present study. First, conceptual semi-distributed modelling using HEC-HMS constitutes a mature and extensively validated approach whose conventional method combination of SCS-CN loss, SCS unit hydrograph transform, and Muskingum routing performs satisfactorily across monsoonal and semi-arid regimes, provided that method selection is justified with reference to catchment characteristics and modelling purpose and that curve number estimation receives particular methodological attention (Hamdan et al., 2021; Horton et al., 2022; Labade et al., 2025; Soulis, 2021). Second, the spatial data infrastructure required for such modelling, comprising global elevation products, satellite land-cover classification, and satellite or reanalysis precipitation, is available for Bangladesh, but low-relief deltaic terrain imposes recognised constraints upon automated delineation and satellite precipitation products retain documented limitations for extreme intensities, both of which require deliberate management (Datta et al., 2022; Pradhan et al., 2022; Uuemaa et al., 2020).

Third, published Bangladeshi applications of HEC-HMS concentrate within large transboundary basins, flashy piedmont catchments, and the Atrai–Karatoa corridor (Jawad, 2024; M. B. Haque et al., 2024; Moniruzzaman & Mahalder, 2026; Nujhat et al., 2024; Nur et al., 2022; S. Haque et al., 2021; Zhang et al., 2022). No published study has developed a rainfall–runoff model for the beel catchments of the moribund Ganges distributaries, of which the Ichamoti–Gumai Beel system is representative. Fourth, the hydrological problem posed by such systems is distinctive in kind rather than merely in degree, since inundation is governed by locally generated runoff accumulating behind congested drainage rather than by riverine flood waves propagating from upstream (Adnan et al., 2020; Parvez et al., 2021), while the long-term deterioration of the distributary network reflects basin-scale flow reduction that has been quantified for the neighbouring Gorai but never connected to a quantitative runoff model of the beel itself (Ali & Hasan, 2022).

The research gap addressed by the present study follows directly from these conclusions. A calibrated and validated HEC-HMS rainfall–runoff model of the Gumai Beel catchment will provide the first quantitative estimate of the runoff volumes and hydrograph dynamics that the Ichamoti drainage corridor is required to convey, thereby establishing the hydrological foundation for drainage design and for evaluation of the ongoing river rejuvenation programme. Methodologically, the study extends the Bangladeshi HEC-HMS literature into an ultra-flat, wetland-dominated, and data-scarce environment in which DEM limitations, seasonal alternation of land cover, and wetland storage behaviour present challenges that have been identified but not resolved in previous work. The methods adopted in the following chapter, comprising comparative evaluation of loss and transform formulations, field-verified catchment delineation, gauge-primary precipitation forcing supplemented by reanalysis data, split-sample testing informed by current calibration research, and performance evaluation using purpose-matched objective functions together with multiple statistical indicators, are each grounded in the literature reviewed in this chapter (Althoff & Rodrigues, 2021; Shen et al., 2022; Turkar et al., 2025).

References

Admas, M., Asrade, T. M., & Cherie, W. D. (2025). Application of the HEC-RAS and HEC-HMS models for flood risk analysis in the Gumara River, Upper Blue Nile Basin, Ethiopia. Advances in Meteorology, 2025, Article 5092932. https://doi.org/10.1155/adme/5092932

Adnan, M. S. G., Talchabhadel, R., Nakagawa, H., & Hall, J. W. (2020). The potential of Tidal River Management for flood alleviation in South Western Bangladesh. Science of the Total Environment, 731, Article 138747. https://doi.org/10.1016/j.scitotenv.2020.138747

Akter, A., & Sawon, F. S. (2024). Hydrological modeling of the selected flash flood-prone rivers. Natural Hazards. https://doi.org/10.1007/s11069-024-06928-z

Ali, M. S., & Hasan, M. M. (2022). Environmental flow assessment of Gorai River in Bangladesh: A comparative analysis of different hydrological methods. Heliyon, 8(7), Article e09857. https://doi.org/10.1016/j.heliyon.2022.e09857

Aliye, M. A., Aga, A. O., Tadesse, T., & Yohannes, P. (2020). Evaluating the performance of HEC-HMS and SWAT hydrological models in simulating the rainfall-runoff process for data scarce region of Ethiopian Rift Valley Lake Basin. Open Journal of Modern Hydrology, 10(4), 105–122. https://doi.org/10.4236/ojmh.2020.104007

Althoff, D., & Rodrigues, L. N. (2021). Goodness-of-fit criteria for hydrological models: Model calibration and performance assessment. Journal of Hydrology, 600, Article 126674. https://doi.org/10.1016/j.jhydrol.2021.126674

Bhattacharjee, S., Islam, M. T., Kabir, M. E., & Kabir, M. M. (2021). Land-use and land-cover change detection in a north-eastern wetland ecosystem of Bangladesh using remote sensing and GIS techniques. Earth Systems and Environment, 5(2), 319–340. https://doi.org/10.1007/s41748-021-00228-3

Datta, S., Karmakar, S., Mezbahuddin, S., Chaudhary, B. S., Hossain, M. M., Hoque, M. E., Abdullah-Al-Mamun, M. M., & Baul, T. K. (2022). The limits of watershed delineation: Implications of different DEMs, DEM resolutions, and area threshold values. Hydrology Research, 53(8), 1047–1062. https://doi.org/10.2166/nh.2022.126

Dibaba, W. T., Demissie, T. A., & Miegel, K. (2020). Watershed hydrological response to combined land use/land cover and climate change in highland Ethiopia: Finchaa catchment. Water, 12(6), Article 1801. https://doi.org/10.3390/w12061801

Hamdan, A. N. A., Almuktar, S., & Scholz, M. (2021). Rainfall-runoff modeling using the HEC-HMS model for the Al-Adhaim River catchment, northern Iraq. Hydrology, 8(2), Article 58. https://doi.org/10.3390/hydrology8020058

Haque, M. B., Karmakar, S., & Hossain, M. M. (2024). Rainfall-runoff modeling using the HEC-HMS flow modeling framework for the Halda River catchment, Bangladesh [Preprint]. Research Square. https://doi.org/10.21203/rs.3.rs-3824469/v1

Haque, S., Ali, M. M., Islam, A. K. M. S., & Khan, M. J. U. (2021). Changes in flow and sediment load of poorly gauged Brahmaputra river basin under an extreme climate scenario. Journal of Water and Climate Change, 12(3), 937–954. https://doi.org/10.2166/wcc.2020.219

Hersbach, H., Bell, B., Berrisford, P., Hirahara, S., Horányi, A., Muñoz-Sabater, J., Nicolas, J., Peubey, C., Radu, R., Schepers, D., Simmons, A., Soci, C., Abdalla, S., Abellan, X., Balsamo, G., Bechtold, P., Biavati, G., Bidlot, J., Bonavita, M., … Thépaut, J.-N. (2020). The ERA5 global reanalysis. Quarterly Journal of the Royal Meteorological Society, 146(730), 1999–2049. https://doi.org/10.1002/qj.3803

Horton, P., Schaefli, B., & Kauzlaric, M. (2022). Why do we have so many different hydrological models? A review based on the case of Switzerland. WIREs Water, 9(1), Article e1574. https://doi.org/10.1002/wat2.1574

Jawad, M. (2024). Evaluation of near real-time Global Precipitation Measurement (GPM) precipitation products for hydrological modelling and flood inundation mapping of sparsely gauged large transboundary basins—A case study of the Brahmaputra basin. Remote Sensing, 16(10), Article 1756. https://doi.org/10.3390/rs16101756

Labade, P., Ayare, B. L., Bhange, H. N., Ingle, P. M., & Kolhe, P. R. (2025). A comprehensive review on rainfall-runoff modelling using HEC-HMS. International Journal of Research in Agronomy, 8(10), 1130–1138. https://doi.org/10.33545/2618060X.2025.v8.i10o.4374

Moniruzzaman, M., & Mahalder, B. (2026). Assessing SWAT and HEC-HMS model efficiency for watershed management in the Atrai-Karatoa River Basin, Bangladesh. Evolving Earth, 4, Article 100136. https://doi.org/10.1016/j.eve.2026.100136

Nujhat, M., Rayhan, M., & Amin, M. K. (2024). Hydrological modelling and its implication in sustainable water resource management in Gumti River Basin in Bangladesh. International Journal of Sustainability in Energy and Environment, 1(2), 40–48.

Nur, F., Mohib, K. M., Toma, U. T., Rozario, P. M., Bari, M. S., Anjum, N., & Khandakar, F. (2022, February 10–12). Rainfall-runoff simulation of Khowai River basin using HEC-HMS model [Conference presentation]. 6th International Conference on Civil Engineering for Sustainable Development (ICCESD 2022), Khulna University of Engineering & Technology, Khulna, Bangladesh.

Parvez, M., Sadat, N., Tasnim, F., & Nejhum, I. J. (2021). Identifying the causes of waterlogging on people’s perception towards a resilient community: A case study on Pabna Municipality, Bangladesh. Ecofeminism and Climate Change, 2(3), 110–126. https://doi.org/10.1108/EFCC-11-2020-0033

Pradhan, R. K., Markonis, Y., Vargas Godoy, M. R., Villalba-Pradas, A., Andreadis, K. M., Nikolopoulos, E. I., Papalexiou, S. M., Rahim, A., Tapiador, F. J., & Hanel, M. (2022). Review of GPM IMERG performance: A global perspective. Remote Sensing of Environment, 268, Article 112754. https://doi.org/10.1016/j.rse.2021.112754

Raihan, F., Beaumont, L. J., Maina, J., Saiful Islam, A. K. M., & Harrison, S. P. (2020). Simulating streamflow in the Upper Halda Basin of southeastern Bangladesh using SWAT model. Hydrological Sciences Journal, 65(1), 138–151. https://doi.org/10.1080/02626667.2019.1682149

Ranjan, S., & Singh, V. P. (2022). HEC-HMS based rainfall-runoff model for Punpun river basin. Water Practice & Technology, 17(5), 986–1001. https://doi.org/10.2166/wpt.2022.033

Sahu, M. K., Shwetha, H. R., & Dwarakish, G. S. (2023). State-of-the-art hydrological models and application of the HEC-HMS model: A review. Modeling Earth Systems and Environment, 9(3), 3029–3051. https://doi.org/10.1007/s40808-023-01704-7

Salimi, S., Almuktar, S. A. A. A. N., & Scholz, M. (2021). Impact of climate change on wetland ecosystems: A critical review of experimental wetlands. Journal of Environmental Management, 286, Article 112160. https://doi.org/10.1016/j.jenvman.2021.112160

Shen, H., Tolson, B. A., & Mai, J. (2022). Time to update the split-sample approach in hydrological model calibration. Water Resources Research, 58(3), Article e2021WR031523. https://doi.org/10.1029/2021WR031523

Shi, W., & Wang, N. (2020). An improved SCS-CN method incorporating slope, soil moisture, and storm duration factors for runoff prediction. Water, 12(5), Article 1335. https://doi.org/10.3390/w12051335

Soulis, K. X. (2021). Soil Conservation Service Curve Number (SCS-CN) method: Current applications, remaining challenges, and future perspectives. Water, 13(2), Article 192. https://doi.org/10.3390/w13020192

Tk 1,554cr project to revive dying Ichamati. (2024, November 26). The Daily Star. https://www.thedailystar.net/news/bangladesh/news/tk-1554cr-project-revive-dying-ichamati-3555931

Turkar, R. K., Shrivastava, R. N., Awasthi, M. K., & Lodhi, A. S. (2025). A comprehensive review of the HEC-HMS rainfall–runoff simulation model and its hydrological applications. Journal of Agriculture and Ecology Research International, 26(5), 130–144. https://doi.org/10.9734/jaeri/2025/v26i5711

U.S. Army Corps of Engineers, Hydrologic Engineering Center. (n.d.). HEC-HMS technical reference manual. Retrieved August 17, 2026, from https://www.hec.usace.army.mil/confluence/hmsdocs/hmstrm

Uuemaa, E., Ahi, S., Montibeller, B., Muru, M., & Kmoch, A. (2020). Vertical accuracy of freely available global digital elevation models (ASTER, AW3D30, MERIT, TanDEM-X, SRTM, and NASADEM). Remote Sensing, 12(21), Article 3482. https://doi.org/10.3390/rs12213482

Yankyera, S., & Alam, B. M. (2025). Five decades of transformation due to human-environment stressors: Land cover, vegetation, and land surface temperature change analysis in the largest wetland ecosystem in Bangladesh. Earth Systems and Environment, 9(2), 589–604. https://doi.org/10.1007/s41748-025-00652-9

Zhang, K., Shalehy, M. H., Ezaz, G. T., Chakraborty, A. K., Mohib, K. M., & Liu, L. (2022). An integrated flood risk assessment approach based on coupled hydrological-hydraulic modeling and bottom-up hazard vulnerability analysis. Environmental Modelling & Software, 148, Article 105279. https://doi.org/10.1016/j.envsoft.2021.105279